Specification for Med Stout grade poles

The poles shall be manufactured and treated in accordance with NPG pole specification NPS/001/001 with the majority of the poles being fabricated to drawings 1000434001 (HV std Fab) & 1091193301 sheet 3 & 4 (LV Std fab). Both drawings to be edited to show new pole grade.

The poles shall be supplied to the following list of dimensions:-

| Pole Type | Pole Height | Min Pole Top Dimension | Min dimension 1.5m from the pole butt |
| --- | --- | --- | --- |
| Med Stout | 10m | 170 | 260 |
| Med Stout | 11m | 175 | 270 |
| Med Stout | 12m | 180 | 285 |
| Med Stout | 13m | 185 | 295 |